# Homework #1

Due 28 August 2021

Solutions

- Consider a point charge held a distance d above an infinite grounded conducting plane, as shown below.

- Sketch the image charges and calculate the potential in the region above the plate.

- What is the force on the charge q?

Solution:

Mathematically, we need to solve Poisson’s equation in the region of interest (z > 0). We can assemble a charge configuration that possesses the same boundary conditions as the grounded conducting plane (V = 0 in the xy-plane), if we place an image charge (same magnitude, opposite sign) at an equal distance on the other side of the xy-plane, as shown below.

For this configuration, we can simply write the potential for z > 0 as a superposition of the potential from each point charge.

Vx,y,z= 14πϵ0qx2+y2+(z-d)2+-qx2+y2+(z+d)2

The force at the charge q can also be written down, but the vector nature must be taken into account.

Fq=14πϵ0-q2(2d)2z

- Consider an infinitely long rectangular slot consisting of two infinite grounded plates that lie parallel to the xy plane. One plate is located at y = 0 and the other is at y = a. The bounded end of the slot consists of an isolated metal strip held at a potential V0(y). See the drawing below.

- Write down the form of Laplace’s equation ….